Numerical Study of the Effect of Long Shroud Diffuser with and without Plate Obstacle on the Performance of Savonius Wind Turbine.

Abstract – This study presents the three-dimensional (3-D) numerical analysis of the influence of the plate obstacle installed at the entrance of the long flanged diffuser on the simple Savonius rotor performance by ANSYS 2022 R1. In the beginning, the 3-D numerical model is validated using the sst k-ω turbulence model with the previous published experimental results at the same conditions, and they are in good agreement with each other. The numerical results indicate the remarkable enhancement of the rotor performance at a tip speed ratio of 0 to 0.7 by using a flanged diffuser with a plate obstacle. The shrouded diffuser with obstacles is better where it reaches a maximum torque coefficient of 0.38 at a tip speed ratio of 0.4, which means a 52% increase compared with the maximum torque coefficient of the un-shrouded rotor at the same conditions. Also, the power coefficient is increased by 20% at a tip speed ratio of 0.5. The shrouded diffuser rotor, with or without obstacles, is not be effective after the tip speed ratio equals to 0.7, where the rotor performance is decreased.

Keywords- Savonius Turbine, Wind Energy, Shroud, CFD.

- Nomenclature

| Cp | Power coefficient (dimensionless) |
| --- | --- |
| Ct | Torque coefficient (dimensionless) |
| T | Torque ( N.m) |
| ω | Angular speed of rotor ( rad/s) |
| ρ | Air density ( kg/m3) |
| U | Wind velocity ( m/s) |
| H | Rotor height (m) |
| D | Rotor diameter (m) |
| Do | Endplate diameter (m) |
| R | Rotor blade radius (m) |
| λ | Tip speed ration (ω.r)/(U) (dimensionless) |
| As | Aspect ratio (H/D) |
| a | Overlap distance (m) |
| G | Overlap ratio ( a/2r) (dimensionless) |

- Introduction

These days, energy is one of the most important and effective elements for the economic growth of most countries. The world's leaders' awareness of climate change and the pollution issues caused by carbon emissions from using fossil fuels puts the development of renewable energy sources under their spotlight.

Wind energy is one of the simplest and oldest sources of renewable energy that people used in different forms in the past. Nowadays, it has evolved into a booming global business, especially in energy generation. The running costs of wind turbines fell in a dramatic way over the last decade, moving closer to those of traditional energy resources [1, 2].

Based on the direction of wind turbine's rotational axis, wind turbines are classified to two categories: vertical-axis wind turbines (VAWT) and horizontal-axis wind turbines (HAWT). The Savonius turbine is categorized as a vertical-axis wind turbine. The profile of its rotors is a simple semi-cylindrical shape that can be easily fabricated from oil drums, [3]. The popular advantages of Savonius rotor are its simple construction, ease of maintenance, and low cost. All of these advantages led to the promising opportunity for Savonius rotors in the electricity business sector, especially in developing countries.

- Literature review

All experimental and numerical studies indicated that the power coefficient (Cp) of Savonius wind turbine is very low compared to others. However, researchers performed more trials to raise up its performance especially at low wind speed conditions.

Every day, more attempts are being conducted by researchers in this field to improve the Savonuis rotor type's performance. There are two ways that this development is going. The first is developing the blade rotor's geometry, gap, overlap, number of rotors, end plate, and other features. The second way is enhancing the augmented objects such as guide boxes, deflectors, obstacles, guide vanes, etc. In this study, the second way is selected for improving the Savonius performance. In the following the relevant work is reviewed and then the aim of this research is presented.

This literature review presents the contributions of researchers in the second direction of Savonius performance enhancement.

The effect of obstacles at different positions on the performance of the ducted Savonius wind turbine was studied numerically and experimentally by Abdelmordy et al. [4]. Their results indicated 31% increase in power coefficient when compared to the conventional value. The Savonius turbine's performance was improved by W. Senarathna et al. [5] used omnidirectional guide vanes in addition to optimized configured blades. According to their numerical analysis, the power captured increased by 80% at a tip speed ratio of 0.8.The performance of a modified blade under different conditions was studied by Z. Mu et al. [6]. They found that the conventional rotor is unable to start the rotation between 120° and 170°, where their modified blade can start at any angle. Additionally, there was a 36% increase in torque coefficient. P. Apparavoo et al. [7] conducted a numerical investigation using two-dimensional (2-D) analysis to examine the influence of guide vanes at different angles on the Savonius wind turbine's performance that installed inside the solar chimney. They found the higher power coefficient of 0.44, which occurred at a 45° angle and increase of power increase about 22% compared with the traditional rotor. T.Ogawa and H.Yoshida [8] investigated the effect of three types of deflecting plates on the performance of a traditional Savonius turbine in a wind tunnel test section with an air speed of approximately 7 m/s. Their results showed that the most effective deflector plate is a simple one, where a 24% increase in power performance was indicated. B. Altan et al. [9] tested Savonius wind turbines with different models of curtains in front of the exit of an open wind tunnel with an air speed of 7m/s. The experimental results had reached the optimum curtain geometry, which had reduced the negative torque and maximized the inlet free stream velocity, increasing the power coefficient to 0.38. K. Irabu and J. Roy [10] improved and enhanced the conventional Savonius power performance using a guide box tunnel. In the wind tunnel's test section, they carried out the experiments on two- and three-blade models. The power coefficient was increased by 1.5 times for the three blades and by 1.24 times for the two blades when compared to the bare one. Mohamed et al. [11] used an optimizer and CFD code coupling to optimize the shape of a Savonius turbine blade that was augmented with a plate obstacle. The k-Ɛ turbulence model was the more suitable model, and its results agreed well with the previous experimental ones. At tip speed ratio 0.7, the optimum blade with obstacle plate had the highest power coefficient "Cp" of 0.389. Mohammadi et al. [12] proposed a new method for optimizing power performance by numerically testing four different nozzle models placed in front of the facing concave blade. Their findings indicated an enhancement of performance, where the power coefficient increased from 0.16 to 0.39.E. Anter and M. Elkhoury [13] discovered that two-dimensional 2-D simulations were less accurate than three-dimensional 3-D ones. The results of the 3-D simulations were verified and showed that the cased turbine's power coefficient could be enhanced to 0.425 at a tip speed ratio of 0.59. Youssef et al. [14] examined how employing upstream and downstream plate deflectors affected on the conventional Savonius's performance. According to their numerical results, at tip speed ratio 1, the power coefficient increased and reached 0.47. A cylinder with a diameter equal to 0.5 of the Savonius cylinder rotor diameter was used as a plate deflector by Yuwono et al. [15]. Their experimental findings indicated an increase in power coefficient at the location of the small cylinder, which was 1.4 times the cylinder diameter away from the rotor. At tip speed ratio 0.65, the maximum power coefficient of 0.2 occurred, which mean a 12% increase over the conventional one. Mohamed et al. [16] used an optimization algorithm in conjunction with CFD to determine the best positions and geometry for the deflectors. Their results showed that the optimum configuration rasied efficiency by 35.5% at tip speed ratio 0.9. Yahya et al. [17] investigated the effects of tilt angle and guide vane blade on Savonius rotor performance through experimental research in the wind tunnel test section. They discovered that the power performance was highest at various wind speeds for six guide vanes with a tilt angle of 60° reached the highest power performance.The pressure distribution of a Savonius rotor using multiple omnidirectional guide vane models was recorded by Siregar et al. [18]. According to their experimental findings, the power coefficient was raised by 25% at tip speed ratio 0.9 when 12 guide vanes were used. Additionally, it was noted that when the guide blade reached 16, the static torque decreased. Shouman et al. [19] investigated numerically the influence of an optimized curtain arrangement with added fins on the rotor performance using ANSYS. When one fin is added and the curtain blades are arranged optimally, the maximum power coefficient of the Savonius wind rotor is increased by approximately 42%.Tian et al. [20] used passive deflector raised the performance of the Savonius rotor. Their numerical findings demonstrated an impressive impact on the Savonius rotors' improved performance. The passive deflector increased the rotor's capability by 24.91%.

From the previous survey, most researchers used fixed objects installed in front of Savonius rotor. Although this technique enhances the rotor performance, the turbine must be fixed at customized position. If the wind direction is changed, it will be difficult to change the position of turbine every time. Also, the rest of researchers used omni- guide vanes that were effective but raised the cost of rotor.

The objective of this research is to focus on and study numerically the effect of the obstacle plate on the performance of the shrouded diffuser with a simple Savonius rotor. The 3-d numerical analysis is performed on two models, a shrouded diffuser rotor with and without an obstacle plate, and compared with an un-shrouded one.

- Physical model

In this study, the models consist of two main parts, simple Savonius rotor in semicircular shape and the flanged shroud with or without obstacle with following specifications.

- Savonius rotor model

As shown in "Figure 1," the Savonuis rotor used in this study has the same geometry and material specifications as those used by Fujisawa and Gotoh [21, 22].

This model has a 300 mm height (H) and a 300 mm rotor diameter (D), so the aspect ratio of this model (As=H/D) equal one. The blade thickness (t) is 2 mm, and the rotor is augmented with end plates of 330 mm diameter (Do). The overlap ratio (G = a/2R) is 0.15 and the gap ratio is zero where the power coefficient is optimal, where (a) and (R) are the overlap distance and rotor radius, respectively.

- Flanged shrouded model with and without plate obstacle.

It is known that the generated power from wind turbines is a function of cubic value of wind speed. So, any increase in wind speed reflects strongly on the turbine output.

The flanged shroud of wind turbines is one of the important techniques used to improve the rotor's performance. El-Zahaby et al. [23] performed a numerical study to get the optimal parameters of the flanged shroud for horizontal axis wind turbine and achieved a good enhancement in generated power by the rotor.

In this research, the effect of the plate obstacle installed in the entrance of long flanged shrouded rotor is studied numerically and compared with the flanged shroud rotor without obstacle as shown in "Figures 2,3". Also, the performance result of bare rotor is taken as a reference for this comparison.

It is shown in "Figure 4" the whole geometry parameters of the tested model, as the following; plate obstacle angle θ1= 25 °, plate obstacle length L1= 300mm, entrance angel θ2= 5 °, entrance length L2=400mm, diffuser angle θ3= 10°, diffuser length L3= 200mm, flange angle θ4= 12° and flange length L4 =100 mm,

Figure 1. Simple Semicircular Savonius Rotor, [21].

Figure 2. Flanged shroud of Savonius rotor, A) without plate obstacle, B) with plate obstacle.

Figure 3. Isometric view of the flanged shroud Savonius rotor with obstacle plate.

Figure 4. Geometry parameters of the flanged shroud of Savonius rotor with plate obstacle.

- Numerical method

In this work, a commercial package ‘‘ANSYS-FLUENT 2022, R1” is used to perform the three dimensional 3-D simulation.

- 1. DOMAIN and boundary conditions

As shown in "Figures 5, 6", the  whole computational domain consists of a fixed domain, which can be represented as an open tunnel with the rotating domain excluded and a rotating domain (circular zone), which is separated by a sliding interface. Four surrounding walls with inlet and outlet sides make up the fixed domain.

In order to avoid the influence of wall turbulence on the accuracy of the numerical simulation, the domain's dimensions (height, width and length) should be (10Dm * 10Dm * 15Dm), with Dm representing the diameter of the model. The tested model is positioned at the center of the rotating domain, 5 Dm from the top and bottom edges of the rectangular domain, and 5 Dm the distance from the inlet and the leading edge of turbine's blade.

- 2. MESH test

The mesh test is validated by five trials of different numbers of grids (1,990,879), (2,347,389),(3,560,256), (4,956,350) and ( 5,916,446) from coarse to fine. The highest torque coefficient (Ct) of is selected as the comparison parameter between models for determining the suitable mesh size. At the same operating conditions, it is noted that the highest Ct values of rotor at number

Figure 5.  Plan view of the numerical domain with boundary conditions.

Figure 6.  Three-dimensional numerical domain.

of mesh elements (4,956,350) and ( 5,916,446) are typical and very closed to the previous published experimental work, [21].

The mesh of the shrouded simple Savonius model has an un-distributed homogenously grid density at the whole domain. The highest grid densities are located in the regions of the rotating domain and downstream where the vortices are generated as shown in "Figure 7". However, the size of mesh in the interface boundary between the rotating and fixed domains must be the same.

Also, the growth rate of inflation's layer around the turbine blade and shroud walls was used with a suitable rate 1.2 to account for changing the mesh size. In this study, five layers of inflation is used as shown in "Figure 8".

- Time step size test

The determination and selection of the time step size is much effective parameter in the numerical model at FLUENT- ANSYS. So, in this simulation the time step sizes are tested from large down to small for getting the optimal and suitable time size.

The model results between rotational speed and time at various time step sizes, ranging from 0.075s to 0.0012s, are displayed in "Figure 9". The results show that 0.0025s is the ideal time size, with results at time sizes smaller than this value being equivalent.

- Model validation

Five turbulence models; standard K-ε, RNG K-ε, Realizable K-ε, SST K-ω and SST transition were examined to find the proper model in this case. The models were investigated at the same conditions of Fuijisawa’s experimental study [22] which are: velocity of free stream  U = 6 m/s and diameter of rotor D and height H are both equal to 0.3m.

“Figure 10" shows the models’ results between torque coefficient and tip speed ratios which were compared with the previous experimental results [21]. The results show that SST K-ω is very close to the published experimental results.

Figure 7. Mesh of fixed and rotating domains.

Figure 8. Mesh inflation layers for rotor blade's wall.

- Results and discussion

In this study the important parameters such as torque T, rotational speed ω, torque coefficient Ct and Power coefficient Cp were analyzed for flanged shroud models with and without plate obstacle and compared with the un-shrouded conventional rotor for determining and obtaining the enhancement for each one. In the following, the pressure and velocity contours are shown and explained at two positions of the rotors for all models. The first position is where the rotor, with its two blades, is facing the free stream, while the other position is in parallel with the free stream.

Figure 9. Rotational speed against time of Savonius rotor at different time step size.

Figure 10. Torque coefficient and tip speed ratio at different turbulence models.

- Pressure contours

It is shown in the flowing "Figures 11, 12 and 13" that the Savonius rotor is working as a drag machine. The pressure reaches its highest value due to the accumulation on the concave and convex surfaces of the blade from the inlet direction which causes the rotation of rotor. Also, it is obtained that the highest pressure zone in the domain for all shrouded rotors increased in proportion to the entrance length of the shroud. Also, the effect of the flange of the shroud is obtained in the pressure contours. There are two high pressure zones at the flanges of shroud, which were formed due to the vortices. The pressure in the entrance was increasing in proportion to the entrance length of the shroud, whereas the un-shrouded rotor was not loaded with any pressure except for the facing area of the leading blade shown in "Figure 11".

In "Figure 12", at part (A) it is obtained that the pressure increased due to the effect of obstacle plate. So the obstacle plate raises the drag force on the concave blade and reduced the drag force on the convex blade.

- Velocity contours

The velocity contours of the three models are shown in "Figures 14, 15, and 16", which indicate a stagnation zone before the concave and convex blades. But, here, it is very clear that the stagnation on the concave surface at all models is greater than on the convex surface, which is the main reason for the drag force difference that causes the rotation.

Figure 11. Pressure contour of un- shrouded rotor at two positions at the rotation.

Figure 12. Pressure contour of shrouded rotor without plate obstacle at two positions at the rotation.

Figure 13. Pressure contour of shrouded rotor with plate obstacle at two positions at the rotation.

Figure 14. Velocity contour of un-shrouded rotor at two positions at the rotation.

Figure 15. Velocity contour of shrouded rotor without plate obstacle at two positions at the rotation.

Figure 16. Velocity contour of shrouded rotor with plate obstacle at two positions at the rotation.

Figure 17. Velocity vectors contour of shrouded rotor with plate obstacle at the first position.

Figure 18. Velocity vectors contour of shrouded rotor with plate obstacle at the second position.

The influence of the obstacle plate is also presented for the velocity contour in "Figure 16" where the flow is accelerated in the pass between the obstacle plate and shroud wall. Also the plate permits the flow to face the returning (Convex blade) for reducing the negative drag force and raising the mean torque.

Also,it is concluded that the increase in velocity through the shroud was generated by the pressure difference between the pressure inlet, which is high, and the low pressure zone after the flange. The acceleration of the free stream is shown in figures 14, 15, 16, 17 and 18, where the effect of the pressure difference on the velocities is very clear.

The rotor torque values (T) against rotational speeds (ω) are extracted from ANSYS-FLUENT result files for three different models of shrouded rotors and un-shrouded rotors. The power and torque coefficients and tip speed ratio are calculated by Eqs. (1), (2), and (3) in all cases to get the curves of the torque and power coefficients against the tip speed ratio.

| Ct = (4 T)/ (ρ U2 D2 H) | (1) |
| --- | --- |
| Cp = (2 T ω)/ (ρ U3 D H) | (2) |
| λ= (ω.D) / (2.U) | (3) |

In "Figure 19", it is shown that the flanged shroud Savonius rotor with plate obstacle has the maximum torque coefficient of 0.38 at a tip speed ratio of 0.409. Also, in "Figure 20", it is shown that the flanged shroud Savonius rotor with plate obstacle has the maximum Cp of 0.18 at λ= 0.5, but it is not effective after a tip speed ratio of 0.7.

Figure 19. Torque coefficient of shrouded and un-shrouded (bare) rotor against tip speed ratio.

Figure 20. Torque coefficient of shrouded and un-shrouded (bare) rotor against tip speed ratio.

- Conclusion

In this research, the numerical study in investigated by ANSYS 2022 to analyze the effect of flanged diffuser Savonuis rotor with and without plate obstacle. The results indicate the following:

- There is a good agreement between the three –dimensional numerical results and the previous published experimental results at the same conditions especially when SST K-ω turbulence  model is used.

- The performance of rotor is enhanced by using the flanged shroud with and without plate obstacle at tip speed ratio from 0.0 to 0.7.

- The flanged shroud rotor with plate obstacle reaches the maximum torque coefficient 0.38 at tip speed ratio 0.4 with increase 52% increase compared with the un-shrouded one.

- Also, it reaches the maximum power coefficient 0.18 at tip speed ratio 0.5 with enhancement 18% enhancement with the bare one.

- The performance of un- shrouded rotor is better than the two types of shrouded rotor after tip speed ratio 0.7.

Finally the shrouded diffuser with plate obstacle enhances the power and torque performance better than the shrouded without.

It is highly recommended to test numerically and experimentally the effect of obstacle angles and obstacle shapes on the performance of simple and twisted rotor in the future work.

References

[1]  IRENA. 2017. "RENEWABLE POWER: SHARPLY FALLING GENERATIONCOSTS."https://www.irena.org//media/Files/IRENA/Agency/Publication/2017/Nov/IRENA_Sharply_falling_costs_2017.

[2]    S.A.Gamel and Y.A. Sultan, "Winds of Power: Data Analysis for the Relationship between Wind Speed, Gust, and Power Output", Journal of Engineering Research (ERJ),Vol. 7 – No. 5, 2023.

[3]    S.J.Savonius, "The S-rotor and its applications", Mech Eng., 53 (5), pp. 333-338, 1931.

[4]  M.A. Abdelmordy, A.M. Abdelsalam, I.M. Sakr, and K.A. Ibrahim, “Numerical and Experimental Investigation of the Ducted Wind Turbine Performance With and Without Obstacle Plates”, ERJ, PART 2, Mech. Eng., Vol. 46, No. 1, pp. 21 - 31, January 2022.

[5]   W. Senarathna, M. Fernando, T. Silva., and C. Abeykoon, “The Design of a Savonius Wind Turbine with Guide Vanes - A Computational Approach”, Proceedings of the 6th International Conference on Energy Harvesting, Storage, and Transfer (EHST22), Niagara Falls, Canada, Paper No. 133 DOI: 10.11159/ehst22.133, June 2022.

[6]  Z. Mu, G. Tong, Z. Xiao, F.Feng, Y. Li, and G.V. Arne, “Study on Aerodynamic Characteristics of a Savonius Wind Turbine with a Modified blade”, Energies, Vol. 15, No. 18, July 2022.

[7]   P. Apparavoo1, S. Roy, and Y. San, “Performance study of savonius style wind turbines in a solar chimney power plant under guided flow environment”, IOP Conf. Series: Earth and Environmental Science, Sci. 1074 012011, 2022.

[8]   T. Ogawa and H. Yoshida, “The Effects of Deflecting plate and Rotor End Plates on Performances of Savonius-Type Wind Turbine”, Bulletin of JSME, Vol. 29, No. 25, pp. 2115-2121, July 1986.

[9]   B.D. Altan, M. Atilgan and A. Ozdamar, “An experimental study on improvement of a Savonius rotor performance with curtaining”, Experimental Thermal and Fluid Science Vol. 32, No. 8, pp. 1673–1678, September 2008.

[10]   K. Irabu and J.N. Roy, “Characteristics of wind power on Savonius

rotor using a guide-box tunnel”, Experimental Thermal and Fluid Science, Vol. 32, No. 2, pp. 580–586, November 2007.

[11]    M.H. Mohamed, G. Janiga, E. Pap, and D. Thevenin, “Optimal blade shape of a modified Savonius turbine using an obstacle shielding the returning blade”, Energy Conversion and Management, Vol. 52, No. 1, pp. 236–242, January 2011.

[12]   M. .Mohammadi, R. Mohammadi, A. Ramadan, and M.H. Mohamed, “Numerical Investigation of Performance Refinement of a Drag Wind Rotor Using Flow Augmentation and Momentum Exchange Optimization”, Energy, Vol. 158, pp. 592-60, September 2018.

[13]  E. Anter and M. Elkhoury, “Parametric sizing optimization process of a casing for a Savonius Vertical Axis Wind Turbine", Renewable Energy, Vol. 136, pp. 127-138, June 2019.

[14]  K.M. Youssef, A.M. El-Kholy, A.M. Hamed, N.A. Mahmoud, A.M. El-Baz, and T.A. Mohamed, “An innovative augmentation technique of savonius wind turbine performance”, Wind Engineering, Vol. 44, No. 1, pp. 1-20, May 2019.

[15] T. Yuwono, G. Sakti, F.N. Aulia, and A.C. Wijaya, “Improving the performance of Savonius wind turbine by installation of a circular cylinder upstream of returning turbine", Alexandria Engineering Journal, Vol. 59, No. 6, pp. 4923–49327, December 2020.

[16] M.H. Mohamed, F. Alqurashi, and D. Thevenin, “Performance enhancement of a Savonius turbine under effect of frontal guiding plates”, Energy Reports, Vol. 7, pp. 6069-6076, November 2021.

[17]  W. Yahya, K. Ziming, W. juan, and M.S. Qurashi, “Influence of tilt angle and the number of guide vane blades towards the Savonius rotor performance", Energy Reports, Vol. 7, pp. 3317-3327, November 2021.

[18] I.H. Siregar, M. Effendy, and A.H.A. Rasyid, “The Effect of the Number of Vanes in the Omnidirectional Guide Vane on Aerodynamics Characteristic of Stationary Swirling Savonius Rotor”, International Journal of Mechanical Engineering and Robotics Research Vol. 10, No. 8, August 2021.

[19]  M.R. Shouman, M.M. Helal, A.A. El-Haroun, "Numerical prediction of improvement of a Savonius rotor performance with curtaining and fin addition on blade", Alexandria Engineering Journal, Vol. 61, No. 12, pp. 10689–10699, December 2022.

[20]  W. Tian, J. Bian, G. Yang, X. Ni, and Z. Mao, "Influence of a passive upstream deflector on the performance of the Savonius wind turbine", Energy Reports, Vol. 8, pp. 7488–7499, November 2022. (Article)

[21]  N. Fuijisawa, "On the torque mechanism of Savonius rotors", Journal of Wind Engineering and Industrial Aerodynamics”, Vol. 40, No. 3, pp. 277-292, 1992.

[22] N. Fuijisawa and F. Gotoh, "Visualization study of the flow in and around a Savonius rotor", Experiments in Fluids”, Vol. 12, pp. 407-412, April 1992.

[23]  A.M. El-Zahaby, A.E. Kabeel, S.S. Elsayed and M.F. Obiaa, "CFD analysis of flow fields for shrouded wind turbine’s diffuser model with different flange angles", Alexandria Engineering Journal, Vol. 56, No. 1, pp. 171-179, March 2017.